Standard Operating Procedures for Radio Systems

Wall Poster 1 / 6

Procedures for Speaking on the Radio

It is important to follow a standard procedure for speaking on the radio.  Only one person can speak at one time, and while doing so he or she prevents anyone else from speaking.

It is important to remember the following principles of radio use:

- Clarity

- Brevity

- Security

Do not use the radio like a telephone. Unlike a telephone, anyone with another radio can hear what you are saying.

Think about what you want to say before transmitting.  The following procedure words (known as ‘prowords’) are some of those commonly used which help in radio conversations:

| Word or Phrase | Meaning |
| --- | --- |
| Affirmative | Yes |
| Break - Break | Interruption to a transmission |
| Correction | What was just transmitted was incorrect – and the correct version is . . . |
| Disregard | This transmission has been made in error – ignore |
| ETA / ETD | Estimated Time of Arrival / Estimated Time of Departure |
| Figures | Numbers to follow |
| From | Originator of the message is…. |
| Go ahead | Similar to “Send” begin your message |
| Go to channel | Change channel (e.g. move to working channel) |
| I will spell | Next word will be spelt out using the phonetic alphabet |
| Message received | Confirm receipt of a message |
| Negative | No |
| Nothing heard | No reply is received from a call |
| Out | End of transmission, no answer is required or expected |
| Over | End of my transmission, invitation to reply |
| Please relay to | Pass message onto certain station(s)/person(s) |
| Radio Check | Request report on radio reception |
| Repeat all after | Repeat all after a certain word or key phrase |
| Repeat all before | Repeat all before a certain word or key phrase |
| Roger | Acknowledge or confirm |
| Say again | Repeat message or part there of |
| Send | Send your message |
| Standby (1,2,3. . .) | Wait on this channel (1, 2,3.. minutes) |
| This Is | Identifies sending station |
| Wait Over or, Wait Out | Instruction to wait on current channel/frequency for less than 1 minute (over; the conversation is not finished) or more (out; the conversation is finished for now, and you will be called back as soon as possible) |

Standard Operating Procedures for Radio Systems

Wall Poster 2 / 6

International Phonetic alphabet

- When using radios for voice communications it is often necessary to spell out an unfamiliar or difficult to pronounce word.  Since many letters sound similar, the International Phonetic Alphabets was developed to reduce confusion.

- All radio users should know this alphabet by heart:

|  | Phonetic | Spoken As |  | Phonetic | Spoken As |
| --- | --- | --- | --- | --- | --- |
| A | Alpha | AL FAH | N | November | NO VEM BER |
| B | Bravo | BRAH VOH | O | Oscar | OSS CAH |
| C | Charlie | CHAR LEE | P | Papa | PAH PAH |
| D | Delta | DELL TAH | Q | Quebec | KEH BECK |
| E | Echo | ECK OH | R | Romeo | ROW ME OH |
| F | Foxtrot | FOXS TROT | S | Sierra | SEE AIR RAH |
| G | Golf | GOLF | T | Tango | TANG GO |
| H | Hotel | HOH TELL | U | Uniform | YOU NEE FORM |
| I | India | IN DEE AH | V | Victor | VIC TAH |
| J | Juliet | JEW LEE ETT | W | Whiskey | WISS KEY |
| K | Kilo | KEY LOH | X | X-ray | ECKS RAY |
| L | Lima | LEE MAH | Y | Yankee | YANK KEY |
| M | Mike | MIKE | Z | Zulu | ZOO LOO |

| Number | Spoken As | Number | Spoken As |
| --- | --- | --- | --- |
| 0 | ZER – O | 6 | SIX |
| 1 | WUN | 7 | SEV – EN |
| 2 | TOO | 8 | AIT |
| 3 | THUH – REE | 9 | NINE – ER |
| 4 | FO – WER | . | DEH – SEE – MAL |
| 5 | FYFE |  |  |

Standard Operating Procedures for Radio Systems

Wall Poster 3 / 6

Radio Procedures

When calling another person or station:

- Ensure no one else is transmitting at the same time.  Wait for ongoing discussions to finish completely before beginning transmission.

- Make your message brief but precise

- Use the standard procedure words

- Do not identify organisations or personnel by name over the radio

- Use call signs instead of personal names

- Use codes instead of places and locations

- To call another radio / user, press the PTT (Press to Talk) button and say: ‘<their call-sign twice> this is <your call-sign> over’.  Then release the PTT button immediately

- To answer a radio call, press the PTT button and say: ‘Hello <their call-sign> this is <your call-sign> over’.  Then release the PTT button immediately

- After hearing the response, you press the PTT button, say your call-sign again, send your message, and end with ‘over’ or ‘out’.  Release the PTT button.

- After the initial call in a conversation, it is normal for each transmission to begin with the call-sign of the person speaking, but local practices vary and this may not be necessary.

- Break the message into sensible passages with clear pauses between

- Maintain clear speech with normal rhythm and moderate volume

- Hold the microphone approximately 5 -10cm from your mouth

- Talk across the microphone, not into it

- Avoid excessive calling.  Use radios for work-related purposes only.

- Never transmit specific security-related information or travel plans or discuss transfer of cash or goods – other people may be listening

Standard Operating Procedures for Radio Systems

Wall Poster 4 / 6

Radio Operator Check List

Weekly Checks:

- Pour water into the earth stake drain pipe (If installed)

- Clean desk and radio equipment

- Cables hanging in areas which can be caught by feet or furniture

- Check that Spare Fuses are taped to side of radio power supply.

Monthly Checks:

These checks are to be carried out on the first working day of each month.

- Battery water level, top-up as necessary

- Power connections to radios

- Cables to batteries

- Battery connections, look for corrosion of terminals

- Cleanliness and condition of battery box

- Antenna connections to radios

- Inspection of all antenna cables

- Ensure nothing is obstructing the cable run

- Look for any damage

- Check antenna system for loose wires or loose antenna, report twisted HF antenna. The two wires should be straight, parallel and upright to the ground. If lying flat to the ground, the radio will not work properly. Report it to the Telecoms Officer immediately!

- General visual check over the whole radio system. Report anything which may need attention to the Telecoms Officer as soon as possible.

Power Supply and Battery Maintenance

- Any missing fuses, damaged equipment or damage to antennas needs to be immediately reported to the Telecoms Officer for replacement or repairs

- Check battery voltage. Voltages will vary slightly depending on the condition of the battery and the accuracy of the meter being used.

- 13.5 volts when Power supply is on and working properly

- 12.5 volts when Power supply is off.  Battery is ok

- If the voltage is less than 11.5 volts, the battery is very flat and the radios should not be used until the battery/s is recharged.

Power Supply or Battery Problem

- You may need to search and find out why the battery is not charging:

- Check the plus is firmly connected to the wall socket

- Check the power is available and turned on

- Check the Automatic Voltage Regulator is connected and turned on

- Check for a blown fuse

- If all looks ok, start the generator to charge the battery/s

- Report any problem to the Telecoms Officer for attention or repairs.

Standard Operating Procedures for Radio Systems

Wall Poster 5 / 6

Channel and Frequency Lists

Fill out this sheet and distribute to all radio locations.

CARE International

| HF Channel Number | Frequency (if applicable) | Remarks |
| --- | --- | --- |

| VHF Channel Number | Frequency (if applicable) | Remarks |
| --- | --- | --- |
|  |  | Eg. Calling / Talking channel |
|  |  | or Repeater Channel |

Other Agencies

| HF Channel Number | Frequency (if applicable) | Remarks |
| --- | --- | --- |

| VHF Channel Number | Frequency (if applicable) | Remarks |
| --- | --- | --- |
|  |  | Eg. Calling / Talking channel |
|  |  | or Repeater Channel |

VHF Repeater Location/s and channel number

| Location | VHF Channel Number | Remarks / Agency |
| --- | --- | --- |

Standard Operating Procedures for Radio Systems

Wall Poster 5 / 6

Call Sign and Location Code Lists

Call Sign List - HF & VHF

| Person / Location | Call Sign | Sel-Call No. | Remarks / Agency |
| --- | --- | --- | --- |

Location Names and Codes

| Location | Code | Remarks |
| --- | --- | --- |